TFT(Thin-Film-Translator) Display with Arduino

Description:  In this project, we will show how  TFT display works and text show in the TFT display.

Hardware Requirement:

- Arduino board

- TFT display-8pin[LED,SCK,SDK,A0,RESET,CS,GND,VCC]

- Jumper wire

- Breadboard

- USB cable

TFT display and it’s Connection:

TFT Display:

Connection:

| 1.8 TFT Display | Wiring to Arduino Uno |
| --- | --- |
| LED | 3.3 V |
| SCK | 13 |
| SDA | 11 |
| A0 or DC | 9 |
| RESET | 8 |
| CS | 10 |
| GND | GND |
| VCC | 5 V |

Code:

#include<TFT.h> #include<SPI.h> #define cs 10 #define dc 9 #define rst 8 // create an instance of the library TFT tft = TFT(cs, dc, rst); void setup() { // put your setup code here, to run once: tft.begin(); // clear the screen with a black background tft.background(0, 0, 0); //set the text size tft.setTextSize(2); } void loop() { // put your main code here, to run repeatedly: // set a random font color tft.stroke(0,255,0); tft.setCursor(0,40); //tft.setCursor(column, row); tft.print("Hellow World!"); }

Output:

Code for the text moving from up to down:

#include<TFT.h> #include<SPI.h> #define cs 10 #define dc 9 #define rst 8 // create an instance of the library TFT tft = TFT(cs, dc, rst); void setup() { // put your setup code here, to run once: tft.begin(); // clear the screen with a black background tft.background(0, 0, 0); //set the text size tft.setTextSize(2); } void loop() { // put your main code here, to run repeatedly: // set a random font color int i=0; for(i;i<100;i+=5) { tft.background(0,0,0); tft.stroke(0,250,0); tft.text("HELLOW",5,i); // (" ",col,row) delay(100); }; }

Overwrite :

When I print one text into the TFT display and then again print another text into the TFT display, it will overwrite into the previous text.

To stop overwriting on TFT display and send the text from the terminal or Bluetooth terminal  , the code is :

#include<TFT.h> #include<SPI.h> #define cs 10 #define dc 9 #define rst 8 String s; // create an instance of the library TFT tft = TFT(cs, dc, rst); void setup() { // put your setup code here, to run once: tft.begin(); Serial.begin(9600); // clear the screen with a black background //set the text size tft.setTextSize(2); } void loop() { // put your main code here, to run repeatedly: if(Serial.available()!=0) { tft.background(0,0,0); s=Serial.readString(); } // set a random font color tft.stroke(0,255,0); tft.setCursor(0,30); tft.print(s); }

Link for TFT basic:

https://create.arduino.cc/projecthub/electropeak/ultimate-beginner-s-guide-to-run-tft-lcd-displays-by-arduino-081006