TFT(Thin-Film-Translator) Display with Arduino

Description:  In this project, we will show how  TFT display works and text show in the TFT display.

Hardware Requirement:

- Arduino board

- TFT display-8pin[LED,SCK,SDK,A0,RESET,CS,GND,VCC]

- Jumper wire

- Breadboard

- USB cable

TFT display and it’s Connection:

TFT Display:

Connection:

| 1.8 TFT Display | Wiring to Arduino Uno |
| --- | --- |
| LED | 3.3 V |
| SCK | 13 |
| SDA | 11 |
| A0 or DC | 9 |
| RESET | 8 |
| CS | 10 |
| GND | GND |
| VCC | 5 V |

Code:

#include<TFT.h> #include<SPI.h> #define cs 10 #define dc 9 #define rst 8 // create an instance of the library TFT tft = TFT(cs, dc, rst); void setup() { // put your setup code here, to run once: tft.begin(); // clear the screen with a black background tft.background(0, 0, 0); //set the text size tft.setTextSize(2); } void loop() { // put your main code here, to run repeatedly: // set a random font color tft.stroke(0,255,0); tft.setCursor(0,40); //tft.setCursor(column, row); tft.print("Hellow World!"); }

Output:

Code for the text moving from up to down:

#include<TFT.h> #include<SPI.h> #define cs 10 #define dc 9 #define rst 8 // create an instance of the library TFT tft = TFT(cs, dc, rst); void setup() { // put your setup code here, to run once: tft.begin(); // clear the screen with a black background tft.background(0, 0, 0); //set the text size tft.setTextSize(2); } void loop() { // put your main code here, to run repeatedly: // set a random font color int i=0; for(i;i<100;i+=5) { tft.background(0,0,0); tft.stroke(0,250,0); tft.text("HELLOW",5,i); // (" ",col,row) delay(100); }; }

Link for TFT basic:

https://create.arduino.cc/projecthub/electropeak/ultimate-beginner-s-guide-to-run-tft-lcd-displays-by-arduino-081006