Temperature and Humidity Sensor DHT11 and show it to TFT Display

Description:  In this project, we will show the temperature and humidity in TFT LCD Display with the help of  DHT11(temperature and humidity sensor).

Hardware Requirement:

- Arduino Uno board

- Breadboard

- Jumper wire

- TFT LCD Display-8pin

- DHT11 (temperature and humidity sensor)

DHT11 (temperature and humidity sensor):

Connection:  Output pin connected to arduino’s pin-2;

TFT display and it’s Connection:

TFT Display:

Connection:

| 1.8 TFT Display | Wiring to Arduino Uno |
| --- | --- |
| LED | 3.3 V |
| SCK | 13 |
| SDA | 11 |
| A0 or DC | 9 |
| RESET | 8 |
| CS | 10 |
| GND | GND |
| VCC | 5 V |

Code:

#include <SimpleDHT.h> #include<TFT.h> #include<SPI.h> #define cs 10 #define dc 9 #define rst 8 // create an instance of the library TFT tft = TFT(cs, dc, rst); // for DHT11, // VCC: 5V or 3V // GND: GND // DATA: 2 int pinDHT11 = 2; SimpleDHT11 dht11(pinDHT11); void setup() { Serial.begin(9600); tft.begin(); // clear the screen with a black background tft.background(0, 0, 0); //set the text size tft.setTextSize(2); } void loop() { // start working... // read without samples. byte temperature = 0; byte humidity = 0; int err = SimpleDHTErrSuccess; if ((err = dht11.read(&temperature, &humidity, NULL)) != SimpleDHTErrSuccess) { return; } Serial.print((int)temperature); Serial.print(" *C, "); Serial.print((int)humidity); Serial.println(" %"); // DHT11 sampling rate is 1HZ. delay(1500); int t=(int)temperature; int h=(int)humidity; tft.stroke(0,250,0); tft.setTextSize(1); tft.setCursor(5,25); tft.print("Temperature :"); tft.setTextSize(2); tft.setCursor(90,20); tft.print(t); tft.setTextSize(2); tft.setCursor(120,20); tft.print("*C"); tft.setTextSize(1); tft.setCursor(5,55); tft.print("Humidity :"); tft.setTextSize(2); tft.setCursor(90,50); tft.print(h); tft.setTextSize(2); tft.setCursor(120,50); tft.print("%"); }

OUTPUT: